MECH TASK-1

Shourya Kadlag
Robo Task Phase
MIT Manipal

# Degrees of freedom

The degrees of freedom or mobility of a mechanism is the minimum number of input parameters required to define the position of every link in the mechanism relative to a reference link. The degrees of freedom of a body depend on its environment. In a 2D frame, there are 3 degrees of freedom: linear motion along the two perpendicular axes (x,y) and rotational motional around the perpendicular axis. In a 3D frame, there are 6 degrees of freedom: linear motion along the 3 perpendicular axes (x,y,z) and rotational motion along the same three axes. They can also be thought of as mediums through which we can control a system.

# Links

A link is an ideal, perfectly rigid body that connects parts of a system. There are two types of links: reference links and moving links. Reference links are fixed in space and serve as a baseline for the position of all other links. Each mechanism generally has one reference link. All other links other than the reference link can move. In a 2D space, if N links are present in total, the maximum possible degree of freedom is 3(N-1).

# Joints

Joints connect links, restrict motion by imposing constraints, and eliminate degrees of freedom. In mechanics, there are two types of joints: Lower Pairs (Full Joints) and Higher Pairs (Half Joints).

## Lower Pairs(j1)

Lower pairs function through surface-surface contact between links. They permit only one degree of freedom while restricting 2 degrees of freedom in a 2D space. They distribute mechanical load over an area and hence have lower chances of wear and tear resulting in longer life.

- Revolute Joint: It allows pure rotational motion around a single axis and prevents linear motion in x and y directions. They are found in door hinges, bicycle wheel axles, elbow joints in robotic arms.

- Prismatic Joint: It allows pure linear sliding along a single fixed axis and prevents rotation and orthogonal translation. They are used in engine pistons and hydraulic press.

- Screw Joint: It couples linear translation and rotational motion together via a defined thread pitch. While it moves in two directions physically, they are mathematically dependent on one input variable, so it maintains one degree of freedom

## Higher Pairs(j2)

The links maintain contact through a point or line. They preserve two degrees of freedom while removing one degree in 2D plane. They involve concentrated stress and are prone to damage under heavy loads.

- Cam and Follower: Contact occurs along a line or at a point on the cam surface. The follower can both roll and slip against the rotating surface while preserving two degrees of freedom relative to each other. An external spring pushes the follower towards the cam. It is used in printing press and textile machines.

- Meshing Gear Teeth: It involves a combination of rolling and sliding motion. Pure rolling occurs at the pitch point of the gears while sliding occurs at all other points. Each point allows only one degree of freedom even though it consists of rolling and sliding.

# Grubler’s Formula

Grubler's formula is an equation which calculates the degrees of freedom (F) or mobility of a planar mechanism.

F=3N-1-2j1-j2

Where F is the mobility, N is the total number of links, j1 is the number of lower pairs and j2 is the number of higher pairs.